1.1 OUTPUT

PROGRAM

clc;

clear;

close all;

% EX NO: 1 Butterworth IIR Filters

% Design and Analysis of a Butterworth IIR Low-Pass Filter Using MATLAB

Fs = 10000;

Fp = 1000;

Fst = 1500;

[N,Wn] = buttord(Fp/(Fs/2),Fst/(Fs/2),1,60);

[b,a] = butter(N,Wn);

% Frequency Response

figure;

freqz(b,a);

title('Frequency Response');

% Impulse Response

figure;

impz(b,a,50);

title('Impulse Response');

% Step Response

figure;

stepz(b,a);

title('Step Response');

% Pole-Zero Plot

figure;

zplane(b,a);

title('Pole-Zero Plot');

fprintf('Minimum Filter Order (N) = %d\n',N);

fprintf('Cutoff Frequency (Wn) = %.4f\n',Wn);