1.2 OUTPUT

PROGRAM

clc;

clear;

close all;

% EX NO: 1

% Butterworth IIR Filters

% DATE:

% 1.2 Design and Analysis of a Butterworth IIR High-Pass Filter Using MATLAB

% Specifications

Fs = 10000;      % Sampling Frequency (Hz)

Fp = 3000;       % Passband Frequency (Hz)

Fst = 2000;      % Stopband Frequency (Hz)

Rp = 1;          % Passband Ripple (dB)

Rs = 60;         % Stopband Attenuation (dB)

% Determine minimum filter order

[N, Wn] = buttord(Fp/(Fs/2), Fst/(Fs/2), Rp, Rs);

% Design Butterworth High-Pass Filter

[b, a] = butter(N, Wn, 'high');

% Frequency Response

figure;

freqz(b, a);

title('Frequency Response');

% Impulse Response

figure;

[h, n] = impz(b, a, 50);

stem(n, h, 'filled');

grid on;

title('Impulse Response');

% Step Response

figure;

stepz(b, a);

title('Step Response');

% Group Delay

figure;

grpdelay(b, a);

title('Group Delay');

% Pole-Zero Plot

figure;

zplane(b, a);

title('Pole-Zero Plot');

% Display Results

fprintf('Minimum Filter Order (N) = %d\n', N);

fprintf('Normalized Cutoff Frequency (Wn) = %.4f\n', Wn);

fprintf('Cutoff Frequency = %.2f Hz\n', Wn*(Fs/2));