PROGRAM

clc;

clear;

close all;

% EX NO: 2

% Chebyshev IIR Filter

% DATE:

% 2.1 Design and Performance Evaluation of a

% Chebyshev Type-I IIR Low-Pass Filter using MATLAB

% Filter Specifications

Wp = 0.4;      % Passband Edge (Normalized)

Ws = 0.55;     % Stopband Edge (Normalized)

Rp = 1;        % Passband Ripple (dB)

As = 40;       % Stopband Attenuation (dB)

% Calculate Minimum Filter Order

[N, Wn] = cheb1ord(Wp, Ws, Rp, As);

% Design Chebyshev Type-I Low-Pass Filter

[b, a] = cheby1(N, Rp, Wn, 'low');

% Display Filter Details

fprintf('Filter Order = %d\n', N);

fprintf('Normalized Cutoff Frequency = %.4f\n', Wn);

% Stability Check

if all(abs(roots(a)) < 1)

disp('System is STABLE');

else

disp('System is UNSTABLE');

end

% ----------------------------

% Frequency Response

% ----------------------------

figure;

freqz(b, a);

title('Frequency Response');

% ----------------------------

% Impulse Response

% ----------------------------

figure;

impz(b, a);

title('Impulse Response');

% ----------------------------

% Group Delay

% ----------------------------

figure;

grpdelay(b, a);

title('Group Delay');

% ----------------------------

% Pole-Zero Plot

% ----------------------------

figure;

zplane(b, a);

title('Pole-Zero Plot');